HAIRINESS REDUCTION OF RING, ROTOR, AIR-JET AND FRICTION SPUN YARNS BY AIR-NOZZLE DURING WINDING

Asis Patnaik , R. S. Rengasamy, V. K. Kothari, S. M. Ishtiaque, A. Ghosh

Department of Textile Technology, Indian Institute of Technology,

New Delhi 110016, India

and

Hemant Punekar

Fluent India Private Limited, Pune 411057, India

## Abstract

In the present work, hairiness of ring, rotor, air-jet and friction spun dref-2 yarns were reduced by passing them through an air-vortex nozzle during yarn winding operation. It has been observed that S3 hairiness values of yarns wound while passing through air-nozzle are nearly 16-30% less than that of corresponding as-spun yarns. Reduction in number of smaller hairs (1 and 2 mm) by air-nozzle is lower compared to that of longer hairs. More the hair presented to the air-nozzle, larger is the reduction in hairiness. Airflow pattern inside the air-nozzle was simulated to understand the actual mechanism of hairiness reduction. A CFD (computational fluid dynamics) model has been developed to simulate the airflow inside the air-nozzle using FLUENT 6.1 software to solve the three-dimensional flow field.